## 1. System Overview

- Type of Pump: Horizontal Centrifugal Pump

- Fluid Handled: Process Water (pH 6.5–8.5, max temp 60°C)

- Purpose: Circulation of process water for cooling and material transport in industrial processing plant

## 2. Daily Checks

| Task | Procedure | Acceptable Range/Criteria | Tools Required | Est. Time (min) | Safety Warnings |
| --- | --- | --- | --- | --- | --- |
| Visual Inspection | Inspect pump, piping, and base for leaks, corrosion, or damage | No visible leaks/damage | Flashlight, mirror | 5 | Wear safety goggles, gloves |
| Noise Level Check | Listen for abnormal sounds (grinding, knocking, squealing) | 65–80 dB (normal operation) | Sound level meter | 3 | Hearing protection if >85 dB |
| Vibration Check | Measure vibration at bearing housing | ≤ 4.5 mm/s RMS (ISO 10816-3) | Vibration meter | 5 | Secure loose clothing |
| Pressure & Flow Check | Verify discharge pressure and flow rate | Within design specs | Pressure gauge, flowmeter | 5 | Avoid contact with moving parts |

## 3. Weekly/Monthly Preventive Maintenance Tasks

| Task | Frequency | Procedure | Lubricant/Spec | Quantity | Tools Required | Est. Time (min) | Safety Warnings |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Lubricate Bearings | Weekly | Apply grease to bearing grease fittings | Lithium-based NLGI #2 | 30g per fitting | Grease gun | 10 | Lockout/tagout, gloves |
| Clean/Replace Filters | Monthly | Remove, inspect, clean or replace suction strainer and inline filters | N/A | N/A | Wrenches, spare filters | 20 | Depressurize system |
| Inspect Mechanical Seals | Monthly | Check for leaks, wear, and alignment | N/A | N/A | Inspection mirror | 10 | Eye protection, gloves |
| Motor Health Check | Monthly | Record amperage, check for overheating, inspect terminal connections | N/A | N/A | Clamp meter, IR thermometer | 15 | Lockout/tagout, insulated tools |

## 4. Corrective Maintenance Procedures

### A. Cavitation

- Symptoms: Unusual noise (gravel/rattling), vibration, reduced flow

- Procedure:

  - Shut down and lockout pump.

  - Inspect suction line for blockages or air leaks.

  - Check fluid level in supply tank.

  - Clean/replace clogged filters.

  - Restart and monitor.

- Tools Required: Wrenches, flashlight, replacement filters

- Est. Time: 30 min

- Safety Warnings: Lockout/tagout, depressurize system

### B. Leaks (Seal or Piping)

- Symptoms: Visible fluid around seal or pipe joints

- Procedure:

  - Shut down and lockout pump.

  - Identify leak source.

  - Tighten fittings or replace gaskets/seals as needed.

  - Clean area and restart pump.

- Tools Required: Wrenches, seal kit, rags

- Est. Time: 45 min

- Safety Warnings: Eye protection, gloves, lockout/tagout

### C. Overheating Motor

- Symptoms: High temperature, tripped overload, burning smell

- Procedure:

  - Shut down and lockout pump.

  - Check for blocked ventilation, clean as needed.

  - Measure motor amperage; compare to nameplate.

  - Inspect for loose connections.

  - Restart and monitor.

- Tools Required: IR thermometer, clamp meter, screwdriver

- Est. Time: 30 min

- Safety Warnings: Lockout/tagout, insulated gloves

## 5. Tools Required

- Grease gun (NLGI #2 compatible)

- Sound level meter

- Vibration meter

- Clamp meter

- IR thermometer

- Wrenches (metric/imperial)

- Screwdrivers

- Inspection mirror

- Flashlight

- Spare filters, seal kits, gaskets

- Rags, cleaning supplies

- Personal Protective Equipment (PPE): gloves, safety goggles, hearing protection

## 6. Maintenance Log Entry

| Date | Technician Name | Work Performed | Status (Completed/Pending) | Next Due Date: 2026-06-05 |
| --- | --- | --- | --- | --- |
|  |  |  |  | 2026-06-05 |

Note: All maintenance activities must comply with plant safety protocols and lockout/tagout procedures.
Total Estimated Annual Maintenance Cost: $49,953.00